FHE:

- Month 1&2:

FHE Selection - CKKS (approximate arithmetic on real/complex numbers, ideal for neural network weights and floating-point-like computation. Most FL papers doing encrypted model aggregation use this).

Library Evaluation - Microsoft SEAL, OpenFHE (for the Embedded /automotive ECU)

Cross-compilation matters because these libraries are usually built for x86 servers, and we’ll need ARM Cortex-A/M or automotive SoC targets.

Deliverables:

Security specification report — documents chosen scheme, parameter set, and formal security-level justification.

Baseline performance bench-marking on hardware — real encrypt/decrypt/multiply latency numbers on your actual target hardware, establishing the performance floor before any optimization.

- Month 3&4:

Encoding & SIMD Packing - 
FHE cipher-texts can hold multiple plain-text slots via CRT (Chinese Remainder Theorem) batching — so instead of encrypting one value per cipher-text, you pack many model weights/gradients into one cipher-text and operate on all of them in parallel (SIMD-style).

This is critical for making per-layer FHE on a MamKANformer-sized model tractable, since naive per-scalar encryption would be prohibitively slow.

NTT Kernel Acceleration - 
Polynomial multiplication (the core FHE operation) is O(n²) naively but O(n log n) via Number Theoretic Transform (NTT) — the finite-field analog of FFT. This step builds/accelerates the NTT kernel, often with ARM NEON intrinsics or GPU offload, since NTT dominates FHE runtime.

Deliverables:

Optimized Packing & Encoding Module — production code for slot packing.

High-Performance NTT Implementation — the accelerated polynomial-multiplication backend everything else depends on.

- Months 5&6:

Cipher-text Serialization & Communication Protocol - 
FHE cipher texts are large (often 10-100x the plain-text size). This step designs compact serialization and fits cipher text transmission into V2X bandwidth/latency constraints (e.g., DSRC/C-V2X message size limits, real-time deadlines for vehicle communication). This is where your federated updates actually get shipped between vehicles/RSUs and the aggregation server.

- Months 7&8:

Homomorphic Aggregator
This is the heart of privacy-preserving FL: performing FedAvg-style weighted averaging directly on encrypted client updates — the server never sees plaintext gradients. For our adaptive layer-wise design, this likely means different layers (Mamba SSM blocks vs. KAN splines vs. attention heads) may have different encryption/aggregation treatment based on sensitivity or size.

Noise Budgeting Tool
Every homomorphic operation (especially multiplication) consumes "noise budget" — the ciphertext accumulates noise until decryption fails. This tool tracks noise growth through your aggregation circuit depth so you know when bootstrapping or rescaling is needed, and prevents silent correctness failures.

Deliverables:

Verified Homomorphic Aggregator — correctness-verified aggregation logic.

Noise Budgeting Tool — instrumentation for noise-budget tracking across the FL rounds.

- Month 9&10:

End-to-End Encryption Pipeline -

Wiring together everything built so far: client-side encryption → serialization → transmission → homomorphic aggregation → decryption → model update, as one integrated pipeline rather than isolated components.

Hardware-in-the-Loop (HiL) Validation-

Testing the full pipeline against real or simulated vehicle hardware/network conditions — not just software simulation. This validates that FHE overhead doesn't break real-time IDS constraints under actual automotive network jitter/bandwidth.

Deliverables:

Integrated Encryption/Decryption API for IDS — the callable API surface the IDS system uses.

Performance & Power Consumption Audit Report — critical for automotive/embedded contexts where power budget is a hard constraint, not just a nice-to-have.

- Months 11&12: