# FedIoV System Architecture

Algorithmic Specification

# Algorithm 1: FedIoV Main Execution Loop

1. Initialize the global model M_G(0) on the central server.

2. For each federated communication round t = 1, 2, ..., T:

3. Server broadcasts global model M_G(t-1) to all N available vehicles.

4. For each vehicle i ∈ {1, ..., N} in parallel:

5. Establish SecureChannel (ECDH key exchange + AES-256 + mutual auth).

6. Compute local model update ΔM_i(t) ← ClientUpdate(i, M_G(t-1)).

7. End For

8. Aggregate updates: M_G(t) ← ServerAggregate({ΔM_i(t)} for i=1 to N).

9. End For

# Algorithm 2: ClientUpdate (Vehicle i, Global Model M_G)

## Phase 1: Multi-Modal Data Fusion

1. Collect data streams:

• CAN frames → 1D Conv + GELU

• V2X beacons → Small MLP

• GPS/IMU → 1D Conv + Max-Pool

• Vision/LiDAR → Tiny MLP (scalars only)

2. Synchronize streams to a 10Hz grid (ZOH discrete / linear interpolation for continuous). Ensure alignment within a ±50ms tolerance.

3. Extract features using a sliding window (length=64, stride=16).

4. Apply FiLM-style gating based on modality reliability.

5. Output fixed 46-dimensional embedding into the KANConvNet.

## Phase 2: Local Training & Personalization

6. Initialize local model M_i(t-1) ← M_G.

7. Optimize hyperparameters θ ← GA_Optimize().

8. For epoch = 1 to 2:

9. Compute L2-regularized loss: L_reg = L_i + λΩ(M_i)

10. Update weights (using class-balanced weights): M_i(t) ← M_i(t-1) - η∇L_i

11. End For

12. Blend for personalization: M_i(t+1) = α·M_G + (1 - α)·M_i(t)

(Note: α → 1 for urban/fleet conformity, α → 0 for rural/specialized specificity).

## Phase 3: Differential Privacy

13. Compute parameter delta: Δθ = M_i(t+1) - M_G.

14. Clip the update to threshold C = 1.0: Δθ ← Δθ · min(1, C / ||Δθ||₂)

15. Add Gaussian noise to ensure privacy: Δθ ← Δθ + N(0, σ²C²I), where σ = 1.2.

16. Return noisy update Δθ to the server (raw signals are never sent).

# Algorithm 3: ServerAggregate (Set of Updates {Δθ_i})

## Stage 1: TOPSIS Filtering (Outlier Removal)

1. For each client update i:

2. Normalize parameters: m_ij = m_ij / √Σ(m_ij²)

3. Apply weights: w_ij = w_j · m_ij

4. Identify Ideal A⁺_j = max(w_ij) and Anti-ideal A⁻_j = min(w_ij) across all parameters j.

5. For each client i:

6. Compute distance to Ideal: S⁺_i = √Σ(w_ij - A⁺_j)²

7. Compute distance to Anti-ideal: S⁻_i = √Σ(w_ij - A⁻_j)²

8. Calculate closeness score: C_i = S⁻_i / (S⁺_i + S⁻_i)

9. Rank and cluster clients by C_i. Discard low-C_i outliers.

## Stage 2: Multi-Krum Aggregation

10. For each remaining cluster:

11. Compute pairwise squared Euclidean distances between all models.

12. Calculate Score_i as the sum of distances to the (n - 2f - 2) nearest neighbors.

13. Select the top-m models (the ones with the lowest scores) into set S_k.

14. Aggregate the final global model: M_G(t) = (1/|C|) Σ_k [ (1/m) Σ_{i∈S_k} M_i(t) ]

15. Return M_G(t).

# Algorithm 4: GA_Optimize (Hyperparameter Tuning)

1. Define search space H = {α, β, e, μ, δ, ω, λ, γ}.

2. Initialize population P_0 with random combinations from H.

3. While improvement in fitness Δf ≥ ε:

4. Evaluate fitness function f(θ) = Accuracy for all θ ∈ P_t.

5. Select top-performing parameter sets (parents).

6. Crossover parent parameters to create offspring.

7. Mutate offspring with probability P_m.

8. Form new population P_{t+1}.

9. End While

10. Deploy best hyperparameters: θ* = argmax f(θ_i).

11. Return θ*.

# Operational Constraints (Threat Model Enforcement)

• Execute all client-side logic under the assumption of an honest-but-curious server and potential network eavesdropping (mitigated by Phase 1 SecureChannel and Phase 3 DP noise).

• Execute all server-side logic under the assumption of Byzantine/poisoning clients (mitigated by TOPSIS and Multi-Krum aggregation).

• Out of Scope/Assumed Safe: Device compromise, physical tampering, side-channels, and zero-day backdoors.